## 🛡️ SSH Negotiation Failures

### 🏛️ The Problem: The "Security Mismatch"

Modern Linux (Ubuntu 22.04+) has deprecated SHA-1 and RSA-1024 because they are mathematically vulnerable. Older Cisco IOS versions still rely on these for their "Key Exchange" (Kex) and "Host Key" algorithms.

The Error: no matching key exchange method found

### 🏗️ Phase 1: The "Manual Override" (Option 1)

Use this if you are on a jump host you don't own, or if you only need to connect once to fix a config. You are manually telling the SSH client to "lower its shield" for one session.

Bash

ssh -o KexAlgorithms=+diffie-hellman-group14-sha1 -o HostKeyAlgorithms=+ssh-rsa cisco@10.199.199.5

### 🏗️ Phase 2: The "Permanent Fix" (Option 2 - Lab Standard)

This is what you just did. It creates a "Policy" on your Ubuntu server that automatically uses the correct settings for your lab subnet.

1. Open (or create) the config file:

Bash

nano ~/.ssh/config

2. Add the following block:

Plaintext

# Apply these settings to all devices in the 10.199.199.x range

Host 10.199.199.*

KexAlgorithms +diffie-hellman-group14-sha1,diffie-hellman-group-exchange-sha1

HostKeyAlgorithms +ssh-rsa

PubkeyAcceptedAlgorithms +ssh-rsa

3. Set correct permissions:

(Linux is strict; if this file is too "public," it will ignore it.)

Bash

chmod 600 ~/.ssh/config

### 🏗️ Phase 3: The "Specialist's Fix" (The Proper Way)

Instead of making your server less secure, make your switch more secure. If the switch hardware supports it, do this on the Cisco CLI:

Code snippet

conf t

# 1. Generate a modern, 2048-bit key

crypto key generate rsa general-keys modulus 2048

# 2. Tell the switch to use modern algorithms

ip ssh server algorithm kex ecdh-sha2-nistp256

ip ssh server algorithm encryption aes256-ctr

end

### 🛡️ The "Real World" Perspective

In a Banking Institution or a high-security environment, the rules are different:

- Audit Compliance: If an auditor sees diffie-hellman-group1-sha1 or RSA-1024 in your network, they will flag it as a High Risk.

- The Workflow: In the real world, we do Phase 3 first. We upgrade the firmware and the keys on the switches to meet modern standards.

- The Exception: You only use Option 2 when you are managing "Legacy" gear (very old switches) that cannot be upgraded but are still critical to the business.

- Netmiko Note: When you start writing your Netmiko scripts, Netmiko actually has a way to handle this automatically in the code using the SSH_CONFIG parameter, but having it fixed at the OS level (your ~/.ssh/config) is much cleaner.